ASW-27B Assembly Check List

(See other side for Dis-Assembly Check List)

- Extend main ramp

- Pump main ramp up 25-30 times

- Insert tail wheel ramp

- Roll fuselage dolly onto ramp

- Check brake fluid level

- Grease wing pins

- Set right hand wing stand

- Flaps set at 1 or 2

- Unlock airbrakes with tool

- Remove aileron and flap locks

- Remove fuselage stabilizer locks

- Insert right wing and set on wing stand

- Insert left wing, held by assistant

- Insert right spar pin then left spar pin

- Insert winglets

- Insert and bolt tailplane assembly

- Remove tail wheel ramp

- Lower main gear

- Check pressure of tires

- Lower main ramp

- Tape wings and tailplane

- Insert TE probe and tape

- Install batteries

- Place parachute/water/food in cockpit

- [Verify O2 system operation]

- Prepare trailer/vehicle for retrieve

- Perform positive control check

- Deploy tow out gear

ASW-27B Dis-Assembly Check List

(See other side for Assembly Check List)

- [Turn off O2 system]

- Remove batteries for charging

- Flaps at 3b if being tied down

- Tie down glider

Disassembly Steps

- Remove tail dolly

- Extend main ramp

- Deploy fuselage dolly

- Insert fuselage into dolly up to red line

- Pump main ramp up 25-30 times

- Retract main gear

- Remove TE probe and insert red RBF flag

- Remove tail plane assembly

- Remove winglets

- Flaps at 1 or 2

- Set right hand wing on stand

- Left wing held by assistant

- Remove main spar pins

- Remove left wing & place on wing dolly

- Remove right wing & place on wing dolly

- Attach fuselage dolly hold downs

- Lock airbrakes with tool at spar end

- Insert aileron and flap locks

- Insert tail wheel ramp

- Roll fuselage into trailer and connect strap

- Close, latch and lock trailer top

- Close, latch and lock fwd trailer hatch